# ST. ANNE’S COLLEGE OF ENGINEERING AND TECHNOLOGY

# ANGUCHETTYPALAYAM

DEPARTMENT COMPUTER SCIENCE ENGINEERING

CS-6412 – MICROPROCESSOR AND MICROCONTROLLER LAB

IV semEster

Name:___________________________________

Year/sem: _______________Branch:_______

Reg. no.:________________________________

EXPT NO:		     	     8086 PROGRAMMING 		DATE:

ADDITION & SUBTRACTION

AIM:

To write an Assembly Language Program (ALP) for performing the addition and subtraction operation of two byte numbers.

APPARATUS REQUIRED:

| SL.NO | ITEM | SPECIFICATION | QUANTITY |
| --- | --- | --- | --- |
| 1. | # Microprocessor kit | 8086 kit | 1 |
| 2. | # Power Supply | +5 V dc | 1 |

PROBLEM STATEMENT:

Write an ALP in 8086 to add and subtract two byte numbers stored in the memory location 1000H to 1003H and store the result  in the memory location 1004H to 1005H.Also provide an instruction in the above program to consider the carry also and store the carry in the memory location 1006H.

ALGORITHM:

(i) 16-bit addition

- Initialize the MSBs of sum to 0

- Get the first number.

- Add the second number to the first number.

- If there is any carry, increment MSBs of sum by 1.

- Store LSBs of  sum.

- Store MSBs of sum.

(ii) 16-bit subtraction

- Initialize the MSBs of difference to 0

- Get the first number

- Subtract  the second number  from the first number.

- If there is any borrow, increment MSBs of difference by 1.

- Store LSBs of  difference

- Store MSBs of difference.

FLOWCHART

#### ADDITION                                                                                           SUBTRACTION

START

START

SET UP COUNTER (CY)

SET UP COUNTER (CARRY)

GET FIRST OPERAND TO A

GET FIRST OPERAND

GET SECOND OPERAND TO       A

SUBTRACT

SECOND OPERAND

FROM MEMORY

IS THERE ANY CY

A = A + B

YES

IS THERE ANY CARRY

YES

COUNTER = COUNTER  + 1

NO

COUNTER = COUNTER  + 1

STORE THE DIFFERENCE

NO

STORE THE SUM

STORE THE CARRY

STOP

STORE THE CARRY

STOP

ADDITION

| PROGRAM | COMMENTS |
| --- | --- |
| MOV CX, 0000H | Initialize counter CX |
| MOV AX,[1200] | Get the first data in AX reg |
| MOV BX, [1202] | Get the second data in BX reg |
| ADD AX,BX | Add the contents of both the regs AX & BX |
| JNC L1 | Check for carry |
| INC CX | If carry exists, increment the CX |
| L1 : MOV [1206],CX | Store the carry |
| MOV [1204], AX | Store the sum |
| HLT | Stop the program |

SUBTRACTION

| PROGRAM | COMMENTS |
| --- | --- |
| MOV CX, 0000H | Initialize counter CX |
| MOV AX,[1200] | Get the first data in AX reg |
| MOV BX, [1202] | Get the second data in BX reg |
| SUB AX,BX | Subtract the contents of BX from AX |
| JNC L1 | Check for borrow |
| INC CX | If borrow exists, increment the CX |
| L1 : MOV [1206],CX | Store the borrow |
| MOV [1204], AX | Store the difference |
| HLT | Stop the program |

ADDITION

| MEMORY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DATA |  |  |  |  |  |  |

SUBTRACTION

| MEMORY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DATA |  |  |  |  |  |  |

MANUAL CALCULATION

RESULT:.

Thus addition & subtraction of two byte numbers are performed and the result is stored.

EXPT NO:		     	     8086 PROGRAMMING 		DATE:

MULTIPLICATION & DIVISION

AIM:

To write an Assembly Language Program (ALP) for performing the multiplication and division operation of 16-bit numbers .

APPARATUS REQUIRED:

| SL.NO | ITEM | SPECIFICATION | QUANTITY |
| --- | --- | --- | --- |
| 1. | # Microprocessor kit | 8086 | 1 |
| 2. | # Power Supply | +5 V dc | 1 |

PROBLEM STATEMENT:

Write an ALP in 8086 MP to multiply two 16-bit binary numbers and store the result in the memory location. Write instructions for dividing the data and store the result.

ALGORITHM:

- Multiplication of 16-bit numbers:

- Get the multiplier.

- Get the multiplicand

- Initialize the product to 0.

- Product = product + multiplicand

- Decrement the multiplier by 1

- If multiplicand is not equal to 0,repeat from step (d) otherwise store the product.

- Division of 16-bit numbers.

- Get the dividend

- Get the divisor

- Initialize the quotient to 0.

- Dividend = dividend – divisor

- If the divisor is greater, store the quotient. Go to step g.

- If dividend is greater, quotient = quotient + 1. Repeat from step (d)

- Store the dividend value as remainder.

FLOWCHART

#### MULTIPLICATION                                                                  DIVISION

Start

Start

Load Divisor & Dividend

Get Multiplier & Multiplicand

MULTIPLICAND

QUOTIENT = 0

REGISTER=00

DIVIDEND =

DIVIDEND-DIVISOR

REGISTER = REGISTER + MULTIPLICAND

QUOTIENT =  QUOTIENT+1

Multiplier=MULTIPLIER – 1

IS

DIVIDEND <

DIVISOR

NO

IS

NO

?

MULTIPLIER

=0?

YES

YES

STORE QUOTIENT

STORE REMAINDER

= DIVIDEND NOW

STORE THE RESULT

STOP

STOP

MULTIPLICATION

| PROGRAM | COMMENTS |
| --- | --- |
| MOV AX,[1200] | Get the first data |
| MOV BX, [1202] | Get the second data |
| MUL BX | Multiply both |
| MOV [1206],AX | Store the lower order product |
| MOV AX,DX | Copy the higher order product to AX |
| MOV [1208],AX | Store the higher order product |
| HLT | Stop the program |

DIVISION

| PROGRAM | COMMENTS |
| --- | --- |
| MOV AX,[1200] | Get the first data |
| MOV DX, [1202] | Get the second data |
| MOV BX, [1204] | Divide the dividend by divisor |
| DIV BX | Store the lower order product |
| MOV [1206],AX | Copy the higher order product to AX |
| MOV AX,DX | Store the higher order product |
| MOV [1208],AX | Stop the program |
| HLT | Get the first data |

MULTIPLICATION

| MEMORY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DATA |  |  |  |  |  |  |

DIVISON

| MEMORY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DATA |  |  |  |  |  |  |

MANUAL CALCULATION

RESULT:.Thus multiplication & division of two byte numbers are performed and the result is stored

EXPT NO:		     	     8086 PROGRAMMING 		DATE:

LARGEST& SMALLEST

AIM:

To write an Assembly Language Program (ALP) to find the largest and smallest number in a given array.

APPARATUS REQUIRED:

| SL.NO | ITEM | SPECIFICATION | QUANTITY |
| --- | --- | --- | --- |
| 1. | # Microprocessor kit | 8086 | 1 |
| 2. | # Power Supply | +5 V dc | 1 |

PROBLEM STATEMENT:

An array of length 10 is given from the location. Find the largest and smallest number and store the result.

ALGORITHM:

- Finding largest number:

- Load the array count in a register C1.

- Get the first two numbers.

- Compare the numbers and exchange if the number is small.

- Get the third number from the array and repeat the process until C1 is 0.

- Finding smallest number:

- Load the array count in a register C1.

- Get the first two numbers.

- Compare the numbers and exchange if the number is large.

- Get the third number from the array and repeat the process until C1 is 0.

FLOWCHART

# LARGEST NUMBER IN AN  ARRAY        SMALLEST NUMBER IN AN  ARRAY

START

START

INITIALIZE COUNT

POINTER MAX  = 0

INITIALIZE COUNT

POINTER MIN  = 0

PONITER  =

POINTER  + 1

PONITER  =

POINTER  + 1

YES

IS MAX  POINTER  ?

IS MIN  POINTER  ?

YES

NO

NO

MAX  = POINTER

MIN  = POINTER

COUNT = COUNT-1

COUNT = COUNT-1

IS COUNT = 0

?

NO

NO

IS COUNT = 0

?

YES

YES

STORE MAXIMUM

STORE MINIIMUM

STOP

STOP

LARGEST

| PROGRAM | COMMENTS |
| --- | --- |
| MOV SI,1200H | Initialize array size |
| MOV CL,[SI] | Initialize the count |
| INC SI | Go to next memory location |
| MOV AL,[SI] | Move the first data in AL |
| DEC CL | Reduce the count |
| L2 : INC SI | Move the SI pointer to next data |
| CMP AL,[SI] | Compare two data’s |
| JNB L1 | If AL > [SI] then go to L1 ( no swap) |
| MOV AL,[SI] | Else move the large number to AL |
| L1 : DEC CL | Decrement the count |
| JNZ L2 | If count is not zero go to L2 |
| MOV DI,1300H | Initialize DI with 1300H |
| MOV [DI],AL | Else store the biggest number in 1300 location |
| HLT | Stop |

SMALLEST

| PROGRAM | COMMENTS |
| --- | --- |
| MOV SI,1200H | Initialize array size |
| MOV CL,[SI] | Initialize the count |
| INC SI | Go to next memory location |
| MOV AL,[SI] | Move the first data in AL |
| DEC CL | Reduce the count |
| L2 : INC SI | Move the SI pointer to next data |
| CMP AL,[SI] | Compare two data’s |
| JB L1 | If AL < [SI] then go to L1 ( no swap) |
| MOV AL,[SI] | Else move the large number to AL |
| L1 : DEC CL | Decrement the count |
| JNZ L2 | If count is not zero go to L2 |
| MOV DI,1300H | Initialize DI with 1300H |
| MOV [DI],AL | Else store the biggest number in 1300 location |
| HLT | Stop |

LARGEST

| MEMORY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DATA |  |  |  |  |  |  |

SMALLEST

| MEMORY |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DATA |  |  |  |  |  |  |

RESULT:.

Thus largest and smallest number is found in a given array.

EXPT NO:		     	     8086 PROGRAMMING 		DATE:

COPYING A STRING

AIM:

To move a string of length FF from source to destination.

ALGORITHM:

- Initialize the data segment .(DS)

- Initialize the extra data segment .(ES)

- Initialize the start of string in the DS. (SI)

- Initialize the start of string in the ES. (DI)

- Move the length of the string(FF) in CX register.

- Move the byte from DS TO ES, till CX=0.

START

CX=length of string, DF=0.

Move a byte from source string (DS) to destination string (ES)

Decrement CX

Check for ZF=1

STOP

Initialize DS,ES,SI,DI

`

`

NO

COPYING A STRING

| PROGRAM | COMMENTS |
| --- | --- |
| MOV SI,1200H | Initialize destination address |
| MOV DI,1300H | Initialize starting address |
| MOV CX,0006H | Initialize array size |
| CLD | Clear direction flag |
| REP MOVSB | Copy the contents of source into destination until count reaches zero |
| HLT | Stop |

| INPUT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MEMORY |  |  |  |  |  |  |
| DATA |  |  |  |  |  |  |

| OUTPUT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MEMORY |  |  |  |  |  |  |
| DATA |  |  |  |  |  |  |

RESULT:

Thus a string of  a particular length is moved from source segment to destination segment.

EXPT NO:		     	     8086 PROGRAMMING 		DATE:

SEARCHING A STRING

AIM:

To scan for a given byte in the string and find the relative address of the byte from the starting location of the string.

ALGORITHM:

- Initialize the extra segment .(ES)

- Initialize the start of string in the ES. (DI)

- Move the number of elements in the string in CX register.

- Move the byte to be searched in the AL register.

- Scan for the byte in ES. If the byte is found ZF=0, move the address pointed by ES:DI to BX.

START

CX=length of the string,    DF=0.

Scan for a particular character specified in AL Register.

Check for ZF=1

STOP

Initialize DS,ES ,SI,DI

Move DI  to BX

NO

SEARCHING FOR A CHARACTER IN THE STRING

| PROGRAM | COMMENTS |
| --- | --- |
| MOV DI,1300H | Initialize destination address |
| MOV SI, 1400H | Initialize starting address |
| MOV CX, 0006H | Initialize array size |
| CLD | Clear direction flag |
| MOV AL, 08H | Store the string to be searched |
| REPNE SCASB | Scan until the string is found |
| DEC DI | Decrement the destination address |
| MOV BL,[DI] | Store the contents into BL reg |
| MOV [SI],BL | Store content of BL in source address |
| HLT | Stop |

| INPUT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MEMORY |  |  |  |  |  |  |
| DATA |  |  |  |  |  |  |

| OUTPUT |  |
| --- | --- |
| MEMORY LOCATION |  |
| DATA |  |

RESULT:

Thus a given byte or word in a string of  a particular length  in the extra segment(destination) is found .

EXPT NO:		     	     8086 PROGRAMMING 		DATE:

FIND AND REPLACE

AIM:

To find a character in the string and replace it with another character.

ALGORITHM:

- Initialize the extra segment .(E S)

- Initialize the start of string in the ES. (DI)

- Move the number of elements in the string in CX register.

- Move the byte to be searched in the AL register.

- Store the ASCII code of the character that has to replace the scanned byte in BL register.

- Scan for the byte in ES. If the byte is not found, ZF≠1 and repeat scanning.

- If the byte is found, ZF=1.Move the content of BL register to ES:DI.

START

CX=length of the string in ES,    DF=0.

DF=0.

Scan for a particular character specified in AL Register.

Check for ZF=1

STOP

Initialize DS, ES, SI, DI

Move  the content of BL to ES:DI

NO

YES

¿

FIND AND REPLACE A CHARACTER IN THE STRING

| PROGRAM | COMMENTS |
| --- | --- |
| MOV DI,1300H | Initialize destination address |
| MOV SI,1400H | Initialize starting address |
| MOV CX, 0006H | Initialize array size |
| CLD | Clear direction flag |
| MOV AL, 08H | Store the string to be searched |
| MOV BH,30H | Store the string to be replaced |
| REPNE SCASB | Scan until the string is found |
| DEC DI | Decrement the destination address |
| MOV BL,[DI] | Store the contents into BL reg |
| MOV [SI],BL | Store content of BL in source address |
| MOV [DI],BH | Replace the string |
| HLT | Stop |

| INPUT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MEMORY |  |  |  |  |  |  |
| DATA |  |  |  |  |  |  |

| OUTPUT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MEMORY |  |  |  |  |  |  |
| DATA |  |  |  |  |  |  |

RESULT:

Thus a given byte or word in a string of  a particular length  in the extra segment(destination) is found and is replaced with another character.

EXPT NO:		     	     8086 INTERFACING         		DATE:

INTERFACING ANALOG TO DIGITAL CONVERTER

AIM:

To write an assembly language program to convert an analog signal into a digital signal using an ADC interfacing.

APPARATUS REQUIRED:

| SL.NO | #### ITEM | SPECIFICATION | QUANTITY |
| --- | --- | --- | --- |
| 1. | # Microprocessor kit | 8086 | 1 |
| 2. | # Power Supply | +5 V dc,+12 V dc | 1 |
| 3. | # ADC Interface board | - | 1 |

PROBLEM STATEMENT:

The program is executed for various values of analog voltage which are set with the help of a potentiometer. The LED display is verified with the digital value that is stored in a memory location.

THEORY:

An ADC usually has two additional control lines: the SOC input to tell the ADC when to start the conversion and the EOC output to announce when the conversion is complete. The following program initiates the conversion process, checks the EOC pin of ADC 0809 as to whether the conversion is over and then inputs the data to the processor. It also instructs the processor to store the converted digital data at RAM location.

ALGORITHM:

- Select the channel and latch the address.

- Send the start conversion pulse.

- Read EOC signal.

- If EOC = 1 continue else go to step (iii)

- Read the digital output.

- Store it in a memory location.

START

FLOW CHART:

SELECT THE CHANNEL AND LATCH ADDRESS

SEND THE START CONVERSION PULSE

IS EOC = 1?

NO

YES

READ THE DIGITALOUTPUT

STORE THE DIGITAL VALUE IN THE MEMORY LOCATION SPECIFIED

STOP

PROGRAM TABLE

| PROGRAM |  | COMMENTS |  |
| --- | --- | --- | --- |
| MOV AL,00 |  | Load accumulator with value for ALE high |  |
| OUT 0C8H,AL |  | Send through output port |  |
| MOV AL,08 |  | Load accumulator with value for ALE low |  |
| OUT 0C8H,AL |  | Send through output port |  |
| MOV AL,01 |  | Store the value to make SOC high in the accumulator |  |
| OUT 0D0H,AL |  | Send through output port |  |
| MOV AL,00 |  | Introduce delay |  |
| MOV AL,00 |  |  |  |
| MOV AL,00 |  |  |  |
| MOV AL,00 |  | Store the value to make SOC low the accumulator |  |
| OUT 0D0H,AL |  | Send through output port |  |
| L1 : IN AL, 0D8H |  | Read the EOC signal from port & check for end of conversion |  |
| AND AL,01 |  |  |  |
| CMP AL,01 |  |  |  |
| JNZ L1 |  | If the conversion is not yet completed, read EOC signal from port again |  |
| IN AL,0C0H |  | Read data from port |  |
| MOV BX,1100 |  | Initialize the memory location to store data |  |
| MOV [BX],AL |  | Store the data |  |
| HLT |  | Stop |  |
| ANALOG VOLTAGE | DIGITAL DATA ON LED DISPLAY |  | HEX CODE IN MEMORY LOCATION |

RESULT:

Thus the ADC was interfaced with 8086 and the given analog inputs were converted into its digital equivalent.

EXPT NO:		     	     8086 INTERFACING         		DATE:

INTERFACING DIGITAL – TO – ANALOG CONVERTER

AIM :

- To write an assembly language program for digital to analog conversion

- To convert digital inputs into analog outputs & To generate different waveforms

APPARATUS REQUIRED:

| SL.NO | #### ITEM | SPECIFICATION | QUANTITY |
| --- | --- | --- | --- |
| 1. | # Microprocessor kit | 8086 Vi Microsystems | 1 |
| 2. | # Power Supply | +5 V, dc,+12 V dc | 1 |
| 3. | # DAC Interface board | - | 1 |

PROBLEM STATEMENT:

The program is executed for various digital values and equivalent analog voltages are measured and also the waveforms are measured at the output ports using CRO.

THEORY:

Since DAC 0800 is an 8 bit DAC and the output voltage variation is between –5v and +5v. The output voltage varies in steps of 10/256 = 0.04 (approximately). The digital data input and the corresponding output voltages are presented in the table. The basic idea behind the generation of waveforms is the continuous generation of analog output of DAC. With 00 (Hex) as input to DAC2 the analog output is –5v. Similarly with FF H as input, the output is +5v. Outputting digital data 00 and FF at regular intervals, to DAC2, results in a square wave of amplitude 5v.Output digital data from 00 to FF in constant steps of 01 to DAC2. Repeat this sequence again and again. As a result a saw-tooth wave will be generated at DAC2 output. Output digital data from 00 to FF in constant steps of 01 to DAC2. Output digital data from FF to 00 in constant steps of 01 to DAC2. Repeat this sequence again and again. As a result a triangular wave will be generated at DAC2 output.

ALGORITHM:

Measurement of analog voltage:

- Send the digital value of DAC.

- Read the corresponding analog value of its output.

Waveform generation:

Square Waveform:

- Send low value (00) to the DAC.

- Introduce suitable delay.

- Send high value to DAC.

- Introduce delay.

- Repeat the above procedure.

Saw-tooth waveform:

- Load low value (00) to accumulator.

- Send this value to DAC.

- Increment the accumulator.

- Repeat step (ii) and (iii) until accumulator value reaches FF.

- Repeat the above procedure from step 1.

Triangular waveform:

- Load the low value (00) in accumulator.

- Send this accumulator content to DAC.

- Increment the accumulator.

- Repeat step 2 and 3 until the accumulator reaches FF, decrement the

accumulator and send the accumulator contents to DAC.

- Decrementing and sending the accumulator contents to DAC.

- The above procedure is repeated from step (i)

FLOWCHART:

START

MEASUREMENT OF ANALOG VOLTAGE 		SQUARE WAVE FORM

START

INTIALISE THE ACCUMULATOR SEND ACC CONTENT TO DAC

SEND THE DIGITALVALUE TO ACCUMULATOR

DELAY

TRANSFER THE ACCUMULATOR CONTENTS TO DAC

LOAD THE ACC WITH MAX VALUE SEND ACC CONTENT TO DAC

READ THE CORRESPONDING ANALOG VALUE

DELAY

STOP

START

TRIANGULAR WAVEFORM

#### SAWTOOTH WAVEFORM

START

INITIALIZE ACCUMULATOR

INITIALIZE ACCUMULATOR

SEND ACCUMULATOR CONTENT TO DAC

SEND ACCUMULATOR CONTENT TO DAC

INCREMENT ACCUMULATOR CONTENT

IS ACC  FF

YES

INCREMENT ACCUMULATOR CONTENT

NO

IS ACC  FF

DECREMENT ACCUMULATOR CONTENT

YES

NO

SEND ACCUMULATOR CONTENT TO DAC

YES

NO

IS ACC  00

MEASUREMENT OF ANALOG VOLTAGE:

| PROGRAM | COMMENTS |
| --- | --- |
| MOV AL,7FH | Load digital value 00 in accumulator |
| OUT C0,AL | Send through output port |
| HLT | Stop |

| DIGITAL DATA | ANALOG VOLTAGE |
| --- | --- |

##### PROGRAM TABLE: Square Wave

| PROGRAM | COMMENTS |
| --- | --- |
| L2 : MOV AL,00H | Load 00 in accumulator |
| OUT C0,AL | Send through output port |
| CALL L1 | Give a delay |
| MOV AL,FFH | Load FF in accumulator |
| OUT C0,AL | Send through output port |
| CALL L1 | Give a delay |
| JMP L2 | Go to starting location |
| L1 : MOV CX,05FFH | Load count value in CX register |
| L3 : LOOP L3 | Decrement until it reaches zero |
| RET | Return to main program |

##### PROGRAM TABLE: Saw tooth Wave

| PROGRAM | COMMENTS |
| --- | --- |
| L2 : MOV AL,00H | Load 00 in accumulator |
| L1 : OUT C0,AL | Send through output port |
| INC AL | Increment contents of accumulator |
| JNZ L1 | Send through output port until it reaches FF |
| JMP L2 | Go to starting location |

##### PROGRAM TABLE: Triangular Wave

| PROGRAM | COMMENTS |
| --- | --- |
| L3 : MOV AL,00H | Load 00 in accumulator |
| L1 : OUT C0,AL | Send through output port |
| INC AL | Increment contents of accumulator |
| JNZ L1 | Send through output port until it reaches FF |
| MOV AL,0FFH | Load FF in accumulator |
| L2 : OUT C0,AL | Send through output port |
| DEC AL | Decrement contents of accumulator |
| JNZ L2 | Send through output port until it reaches 00 |
| JMP L3 | Go to starting location |

WAVEFORM GENERATION:

| WAVEFORMS | AMPLITUDE | TIMEPERIOD |
| --- | --- | --- |
| ##### Square Waveform |  |  |
| Saw-tooth waveform |  |  |
| Triangular waveform |  |  |

MODEL GRAPH:

Square Waveform							Saw-tooth waveform

Triangular waveform

RESULT:Thus the DAC was interfaced with 8085 and different waveforms have been generated.

EXP.NO: 		STEPPER MOTOR INTERFACING                               DATE:

AIM:

To write an assembly language program in 8086 to rotate the motor at different speeds.

APPARATUS REQUIRED:

| SL.NO | #### ITEM | SPECIFICATION | QUANTITY |
| --- | --- | --- | --- |
| 1. | # Microprocessor kit | 8086 | 1 |
| 2. | # Power Supply | +5 V, dc,+12 V dc | 1 |
| 3. | # Stepper Motor Interface board | - | 1 |
| 4. | # Stepper Motor | - | 1 |

PROBLEM STATEMENT:

Write a code for achieving a specific angle of rotation in a given time and particular number of rotations in a specific time.

THEORY:

A motor in which the rotor is able to assume only discrete stationary angular position is a stepper motor. The rotary motion occurs in a stepwise manner from one equilibrium position to the next.Two-phase scheme: Any two adjacent stator windings are energized. There are two magnetic fields active in quadrature and none of the rotor pole faces can be in direct alignment with the stator poles. A partial but symmetric alignment of the rotor poles is of course possible.

ALGORITHM:

For running stepper motor clockwise and anticlockwise directions

- Get the first data from the lookup table.

- Initialize the counter and move data into accumulator.

- Drive the stepper motor circuitry and introduce delay

- Decrement the counter is not zero repeat from step(iii)

- Repeat the above procedure both for backward and forward directions.

SWITCHING SEQUENCE OF STEPPER MOTOR:

| MEMORY LOCATION | A1 | A2 | B1 | B2 | HEX CODE |
| --- | --- | --- | --- | --- | --- |
| 4500 | 1 | 0 | 0 | 0 | 09 H |
| 4501 | 0 | 1 | 0 | 1 | 05 H |
| 4502 | 0 | 1 | 1 | 0 | 06 H |
| 4503 | 1 | 0 | 1 | 0 | 0A H |

START

FLOWCHART:

INTIALIZE COUNTER FOR LOOK UP TABLE

GET THE FIRST DATA FROM THE  ACCUMULATOR

MOVE DATA INTO THE ACCUMULATOR

DRIVE THE MOTOR CIRCUITARY

DELAY

DECREMENT COUNTER

IS  B = 0  ?

YES

GET THE DATA FROM LOOK UP TABLE

NO

PROGRAM TABLE

| PROGRAM | COMMENTS |
| --- | --- |
| START : MOV DI, 1200H | Initialize memory location to store the array of number |
| MOV CX, 0004H | Initialize array size |
| LOOP 1 : MOV AL,[DI] | Copy the first data in AL |
| OUT 0C0,AL | Send it through port address |
| MOV DX, 1010H | Introduce delay |
| L1 : DEC DX |  |
| JNZ L1 |  |
| INC DI | Go to next memory location |
| LOOP LOOP1 | Loop until all the data’s have been sent |
| JMP START | Go to start location for continuous rotation |
| 1200 : 09,05,06,0A | Array of data’s |

RESULT: Thus the assembly language program for rotating stepper motor in both clockwise and anticlockwise directions is written and verified.

EXP.NO:	INTERFACING PRGRAMMABLE KEYBOARD AND DISPLAY				CONTROLLER- 8279

DATE:

AIM :

To display the rolling message “ HELP US “ in the display.

APPARATUS REQUIRED:

8086 Microprocessor kit, Power supply, Interfacing board.

ALGORITHM :

Display of rolling message “ HELP US “

  - Initialize the counter

  - Set 8279 for 8 digit character display, right entry

  - Set 8279 for clearing the display

  - Write the command to display

  - Load the character into accumulator and display it

  - Introduce the delay

  - Repeat from step 1.

1. Display Mode Setup: Control  word-10 H

- 0                0                1                 0                 0                0                 0

| 0 | 0 | 0 | D | D | K | K | K |
| --- | --- | --- | --- | --- | --- | --- | --- |

DD

00-  8Bit character display  left entry

01- 16Bit character display left entry

10-  8Bit character display right entry

11- 16Bit character display right entry

KKK- Key Board Mode

000-2Key lockout.

2.Clear Display: Control word-DC H

1                 1               0                  1                  1               1                 0                0

| 1 | 1 | 0 | CD | CD | CD | CF | CA |
| --- | --- | --- | --- | --- | --- | --- | --- |

1-Enables Clear display

0-Contents of RAM will be displayed

1-FIFO Status is cleared

1-Clear all bits

(Combined effect of CD)

11             A0-3; B0-3 =FF

3. Write Display: Control word-90H

1                0             0              1                 0               0                 0        0

| 1 | 0 | 0 | ##### AI | ##### A | ##### A | ##### A | ##### A |
| --- | --- | --- | --- | --- | --- | --- | --- |

Selects one of the 16 rows of display.

Auto increment = 1, the row address selected will be incremented after each of read and write operation of the display RAM.

FLOWCHART:

SET UP POINTER

INITIALIZE THE COUNTER

SET 8279 FOR 8-DIGIT CHARACTER DISPLAY

SET 8279 FOR CLEARING THE DISPLAY

WRITE THE COMMAND TO DISPLAY

LOAD THE CHARACTER INTO ACCUMULATOR AND DISPLAY

DELAY

PROGRAM TABLE

| PROGRAM | COMMENTS |
| --- | --- |
| START : MOV SI,1200H | Initialize array |
| MOV CX,000FH | Initialize array size |
| MOV AL,10 | Store the control word for display mode |
| OUT C2,AL | Send through output port |
| MOV AL,CC | Store the control word to clear display |
| OUT C2,AL | Send through output port |
| MOV AL,90 | Store the control word to write display |
| OUT C2,AL | Send through output port |
| L1 : MOV AL,[SI] | Get the first data |
| OUT C0,AL | Send through output port |
| CALL DELAY | Give delay |
| INC SI | Go & get next data |
| LOOP L1 | Loop until all the data’s have been taken |
| JMP START | Go to starting location |
| DELAY : MOV DX,0A0FFH | Store 16bit count value |
| LOOP1 : DEC DX | Decrement count value |
| JNZ LOOP1 | Loop until count values becomes zero |
| RET | Return to main program |

LOOK-UP TABLE:

| 1200 | 98 | 68 | 7C | C8 |
| --- | --- | --- | --- | --- |
| 1204 | FF | 1C | 29 | FF |

| MEMORY LOCATION | 7-SEGMENT LED FORMAT |  |  |  |  |  |  |  | HEX DATA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | d | c | b | a | dp | e | g | f |  |
| 1200H | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 98 |
| 1201H | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 68 |
| 1202H | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 7C |
| 1203H | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | C8 |
| 1204H | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | FF |
| 1205H | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1C |
| 1206H | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 29 |
| 1207H | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | FF |

RESULT:

Thus the rolling message “HELP US” is displayed using 8279 interface kit.

EXP. NO:			INTERFACING USART 8251                              DATE:

AIM:

To study interfacing technique of 8251 (USART) with microprocessor 8086 and write an 8086 ALP to transmit and receive data between two serial ports with RS232 cable.

APPARATUS REQUIRED:

8086 kit (2 Nos), RS232 cable.

THEORY:

The 8251 is used as a peripheral device for serial communication and is programmed by the CPU to operate using virtually any serial data transmission technique. The USART accepts data characters from the CPU in parallel format and then converts them into a continuous serial data stream for transmission. Simultaneously, it can receive serial data streams and convert them into parallel data characters for the CPU. The CPU can read the status of the USART at any time. These include data transmission errors and control signals. The control signals define the complete functional definition of the 8251. Control words should be written into the control register of 8251.These control words are split into two formats: 1) Mode instruction word & 2) Command instruction word. Status word format is used to examine the error during functional operation.

1...transmit enable

1...data terminal ready

1...  receive enable

1...  send break character

1....  reset error flags (pe,oe,fe)

1.....  request to send (rts)

1......  internal reset

1.......  enter hunt mode (enable search for sync characters)

1  ransmitter ready

1.  receiver ready

1..  transmitter empty

1...  parity error (pe)

1....  overrun error (oe)

1.....  framing error (fe), async only

1......  sync detect, sync only

1.......  data set ready (dsr)

ALGORITHM:

- Initialize 8253 and 8251 to check the transmission and reception of a character

- Initialize8253 to give an output of 150Khz at channel 0 which will give a 9600 baud rate of 8251.

- The command word and mode word is written to the 8251 to set up for subsequent operations

- The status word is read from the 8251 on completion of a serial I/O operation, or when the host CPU is checking the status of the device before starting the next I/O operation

START

FLOW CHART:

Check TX/RX Ready

No

Is it High

Yes

Write Data into data register

STOP

PROGRAM: TRANSMITTER END

| PROGRAM | COMMENTS |
| --- | --- |
| MOV AL,36 | Initialize 8253 in mode 3 square wave generator |
| OUT CE,AL | Send through port address |
| MOV AL,10 | Initialize AL with lower value of count (clock frequency 150KHz) |
| OUT C8,AL | Send through port address |
| MOV AL,00 | Initialize AL with higher value of count |
| OUT C8,AL | Send through port address |
| MOV AL,4E | Set mode for 8251(8bit data, No parity, baud rate factor 16x & 1 stop bit) |
| OUT C2,AL | Send through port address |
| MOV AL,37 | Set command instruction(enables transmit enable & receive enable bits) |
| OUT C2,AL | Send through port address |
| L1:IN AL,C2 | Read status word |
| AND AL,04 | Check whether transmitter ready |
| JZ L1 | If not wait until transmitter becomes ready |
| MOV AL,41 | Set the data as 41 |
| OUT C0,AL | Send through port address |
| INT 2 | Restart the system |

RECEIVER END

| PROGRAM | COMMENTS |
| --- | --- |
| MOV AL,36 | Initialize 8253 in mode 3 square wave generator |
| OUT CE,AL | Send through port address |
| MOV AL,10 | Initialize AL with lower value of count (clock frequency 150KHz) |
| OUT C8,AL | Send through port address |
| MOV AL,00 | Initialize AL with higher value of count |
| OUT C8,AL | Send through port address |
| MOV AL,4E | Set mode for 8251(8bit data, No parity, baud rate factor 16x & 1 stop bit) |
| OUT C2,AL | Send through port address |
| MOV AL,37 | Set command instruction(enables transmit enable & receive enable bits) |
| OUT C2,AL | Send through port address |
| L1:IN AL,C2 | Read status word |
| AND AL,02 | Check whether receiver ready |
| JZ L1 | If not wait until receiver becomes ready |
| IN AL,C0 | If it is ready, get the data |
| MOV BX,1500 | Initialize BX register with memory location to store the data |
| MOV [BX],AL | Store the data in the memory location |
| INT 2 | Restart the system |

RESULT:

Thus ALP for serial data communication using USART 8251 is written and the equivalent ASCII 41 for character ‘A’ is been transmitted & received.

# EXP. NO:			INTERFACING PPI 8255                              DATE:

AIM:

To write ALP by interfacing 8255 with 8086 in mode 0, mode 1 and mode 2

APPARATUS REQUIRED:

8086 kit, 8255 interface kit.

ALGORITHM:

Mode 0

- Initialize accumulator to hold control word

- store control word in control word register

- Read data port A.

- Store data from port A in memory

- Place contents in port B

Mode 1 & Mode 2

- Initialize accumulator to hold control word (for port A)

- Store control word in control word register

- Initialize accumulator to hold control word (for port B)

- Place contents in control word register.

- Disable all maskable interrupts, enable RST 5.5

- send interrupt mask for RST 6.5 & 7.5

- Enable interrupt flag

- Read data from port A, place contents in port B

FLOWCHART

START

Mode 0                                                                          Mode 1 & 2

START

Store control word in control register

Store control word in control register

Input to be read from port A

Input to be read from port A

Disable all interrupts except RST 6.5

Store into accumulator

Store output to port B

Output written on port B

STOP

STOP

##### MODE 0

| PROGRAM | COMMENTS |
| --- | --- |
| MOV AL,90H | Set the control word |
| OUT C6,AL | Send it to control port |
| IN AL,C0 | Get the contents of port A in AL |
| OUT C2,AL | Send the contents of port B to port address |
| HLT | Stop |

##### MODE 1

| PROGRAM | COMMENTS |
| --- | --- |
| MOV AL,0B0H | Set the control word for mode 1 |
| OUT C6,AL | Send it to control port |
| MOV AL,09H | Control for BSR mode |
| OUT C6,AL | Send it to control port |
| MOV AL,13H | Interrupt generation |
| OUT 30,AL |  |
| MOV AL,0AH | Through 8259 |
| OUT 32,AL |  |
| MOV AL,0FH | Using IR2 interrupt(lower order count) |
| OUT 32,AL |  |
| MOV AL,00H | Higher order count |
| OUT 32,AL |  |
| STI | Set trap flag |
| HLT | Stop |
| ISR: | Subroutine |
| IN AL,C0 | Read from Port A |
| OUT C2,AL | Send it to Port B |
| HLT | Stop |

##### MODE 2

| PROGRAM | COMMENTS |
| --- | --- |
| MOV AL,0C0H | Set the control word for mode 2 |
| OUT C6,AL | Send it to control port |
| MOV AL,09H | Control for BSR mode |
| OUT C6,AL | Send it to control port |
| MOV AL,13H | Interrupt generation |
| OUT 30,AL |  |
| MOV AL,0AH | Through 8259 |
| OUT 32,AL |  |
| MOV AL,0FH | Using IR2 interrupt(lower order count) |
| OUT 32,AL |  |
| MOV AL,00H | Higher order count |
| OUT 32,AL |  |
| STI | Set trap flag |
| HLT | Stop |
| ISR: | Subroutine |
| IN AL,C0 | Read from Port A |
| OUT C2,AL | Send it to Port B |
| HLT | Stop |

## BSR mode

Bit set/reset, applicable to PC only. One bit is S/R at a time. Control word:

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 (0=BSR) | X | X | X | B2 | B1 | B0 | S/R (1=S,0=R) |

Bit select: (Taking Don't care's as 0)

| B2 | B1 | B0 | PC bit | Control word (Set) | Control word (reset) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0000 0001 = 01h | 0000 0000 = 00h |
| 0 | 0 | 1 | 1 | 0000 0011 = 03h | 0000 0010 = 02h |
| 0 | 1 | 0 | 2 | 0000 0101 = 05h | 0000 0100 = 04h |
| 0 | 1 | 1 | 3 | 0000 0111 = 07h | 0000 0110 = 06h |
| 1 | 0 | 0 | 4 | 0000 1001 = 09h | 0000 1000 = 08h |
| 1 | 0 | 1 | 5 | 0000 1011 = 0Bh | 0000 1010 = 0Ah |
| 1 | 1 | 0 | 6 | 0000 1101 = 0Dh | 0000 1100 = 0Ch |
| 1 | 1 | 1 | 7 | 0000 1111 = 0Fh | 0000 1110 = 0Eh |

## I/O mode

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 (1=I/O) | GA mode select |  | PA | PCU | GB mode select | PB | PCL |

- D6, D5: GA mode select:

  - 00 = mode0

  - 01 = mode1

  - 1X = mode2

- D4(PA), D3(PCU): 1=input 0=output

- D2: GB mode select: 0=mode0, 1=mode1

- D1(PB), D0(PCL): 1=input 0=output

Mode 0                           Mode 1                         Mode 2

| Input | Output | Input | Output | Input | Output |
| --- | --- | --- | --- | --- | --- |

Result:

The programs for interfacing 8255 with 8085 are executed & the output is obtained for modes 0,1 & 2

EXPT NO:		                8051   PROGRAMMING	     		  DATE:

8 BIT ADDITION

AIM:

To write a program to add two 8-bit numbers using 8051 microcontroller.

ALGORITHM:

1. Clear Program Status Word.

2. Select Register bank by giving proper values to RS1 & RS0 of PSW.

3. Load accumulator A with any desired 8-bit data.

4. Load the register R 0 with the second 8- bit data.

5. Add these two 8-bit numbers.

6. Store the result.

7. Stop the program.

FLOW CHART:

START

Clear PSW

Select Register Bank

Load A and R 0  with 8- bit datas

Add A & R 0

Store the sum

STOP

8 Bit Addition (Immediate Addressing)

| ADDRESS | LABEL | MNEMONIC | OPERAND | HEX CODE | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | CLR | C | C3 | Clear CY Flag |
| 4101 |  | MOV | A, data1 | 74,data1 | Get the data1 in Accumulator |
| 4103 |  | ADDC | A, # data 2 | 24,data2 | Add the data1 with data2 |
| 4105 |  | MOV | DPTR, # 4500H | 90,45,00 | Initialize the memory location |
| 4108 |  | MOVX | @ DPTR, A | F0 | Store the result in memory location |
| 4109 | L1 | SJMP | L1 | 80,FE | Stop the program |

| OUTPUT |  |
| --- | --- |
| MEMORY LOCATION | DATA |
| 4500 |  |

RESULT:

Thus the 8051 ALP for addition of two 8 bit numbers is executed.

EXPT NO:		8 BIT SUBTRACTION                                                  DATE:

AIM:

To perform subtraction of two 8 bit data and store the result in memory.

ALGORITHM:

- Clear the carry flag.

- Initialize the register for borrow.

- Get the first operand into the accumulator.

- Subtract the second operand from the accumulator.

- If a borrow results increment the carry register.

- Store the result in memory.

FLOWCHART:

START

CLEAR CARRY FLAG

GET I’ST OPERAND IN ACCR

SUBTRACT THE 2’ND OPERAND FROM ACCR

STORE  RESULT IN MEMORY

STOP

IS CF=1

INCREMENT THE BORROW REGISTER

Y

N

8 Bit Subtraction (Immediate Addressing)

| ADDRESS | LABEL | MNEMONIC | OPERAND | HEX CODE | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | CLR | C | C3 | Clear CY flag |
| 4101 |  | MOV | A, # data1 | 74, data1 | Store data1 in accumulator |
| 4103 |  | SUBB | A, # data2 | 94,data2 | Subtract data2 from data1 |
| 4105 |  | MOV | DPTR, # 4500 | 90,45,00 | Initialize memory location |
| 4108 |  | MOVX | @ DPTR, A | F0 | Store the difference in memory location |
| 4109 | L1 | SJMP | L1 | 80,FE | Stop |

| OUTPUT |  |
| --- | --- |
| MEMORY LOCATION | DATA |
| 4500 |  |

RESULT:

Thus the 8051 ALP for subtraction of two 8 bit numbers is executed.

EXPT NO:		              8051   PROGRAMMING	                               DATE:

8 BIT MULTIPLICATION

AIM:

To perform multiplication of two 8 bit data and store the result in memory.

ALGORITHM:

- Get the multiplier in the accumulator.

- Get the multiplicand in the B register.

- Multiply A with B.

- Store the product in memory.

FLOWCHART:

START

GET MULTIPLIER IN ACCR

GET MULTIPLICAND IN B REG

MULTIPLY A WITH B

STORE  RESULT IN MEMORY

STOP

8 Bit Multiplication

| ADDRESS | LABEL | MNEMONIC | OPERAND | HEX CODE | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | MOV | A ,#data1 | 74, data1 | Store data1 in accumulator |
| 4102 |  | MOV | B, #data2 | 75,data2 | Store data2 in B reg |
| 4104 |  | MUL | A,B | F5,F0 | Multiply both |
| 4106 |  | MOV | DPTR, # 4500H | 90,45,00 | Initialize memory location |
| 4109 |  | MOVX | @ DPTR, A | F0 | Store lower order result |
| 401A |  | INC | DPTR | A3 | Go to next memory location |
| 410B |  | MOV | A,B | E5,F0 | Store higher order result |
| 410D |  | MOV | @ DPTR, A | F0 |  |
| 410E | STOP | SJMP | STOP | 80,FE | Stop |

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| MEMORY LOCATION | DATA | MEMORY LOCATION | DATA |
| 4500 |  | 4502 |  |
| 4501 |  | 4503 |  |

RESULT:

Thus the 8051 ALP for multiplication of two 8 bit numbers is executed.

EXPT NO:		              8051   PROGRAMMING	     		  DATE:

8 BIT DIVISION

AIM:

To perform division of two 8 bit data and store the result in memory.

ALGORITHM:

- Get the Dividend in the accumulator.

- Get the Divisor in the B register.

- Divide A by B.

- Store the Quotient and Remainder in memory.

FLOWCHART:

START

GET DIVIDEND IN ACCR

GET DIVISOR IN B REG

DIVIDE A BY B

STORE QUOTIENT & REMAINDER IN MEMORY

STOP

8 Bit Division

| ADDRESS | LABEL | MNEMONIC | OPERAND | HEX CODE | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | MOV | A, # data1 | 74,data1 | Store data1 in accumulator |
| 4102 |  | MOV | B, # data2 | 75,data2 | Store data2 in B reg |
| 4104 |  | DIV | A,B | 84 | Divide |
| 4015 |  | MOV | DPTR, # 4500H | 90,45,00 | Initialize memory location |
| 4018 |  | MOVX | @ DPTR, A | F0 | Store remainder |
| 4109 |  | INC | DPTR | A3 | Go to next memory location |
| 410A |  | MOV | A,B | E5,F0 | Store quotient |
| 410C |  | MOV | @ DPTR, A | F0 |  |
| 410D | STOP | SJMP | STOP | 80,FE | Stop |

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| MEMORY LOCATION | DATA | MEMORY LOCATION | DATA |
| 4500 (dividend) |  | 4502 (remainder) |  |
| 4501 (divisor) |  | 4503 (quotient) |  |

RESULT:

Thus the 8051 ALP for division of two 8 bit numbers is executed.

EXP. NO:		LOGICAL AND BIT MANIPULATION                         DATE:

AIM:

To write an ALP to perform logical and bit manipulation operations using 8051 microcontroller.

APPARATUS REQUIRED:

8051 microcontroller kit

ALGORITHM:

- Initialize content of accumulator as FFH

- Set carry flag (cy = 1).

- AND bit 7 of accumulator with cy and store PSW format.

- OR bit 6 of PSW and store the PSW format.

- Set bit 5 of SCON.

- Clear bit 1 of SCON.

- Move SCON.1 to carry register.

- Stop the execution of program.

FLOWCHART:

START

Set CY flag, AND CY with MSB of ACC

Store the PSW format, OR CY with bit 2 IE reg

Clear bit 6 of PSW, Store PSW

Set bit 5 of SCON , clear bit 1 and store SCON

Move bit 1 of SCON to CY and store PSW

STOP

PROGRAM TABLE

| ADDRESS | HEX CODE | LABEL | MNEMONICS | OPERAND | COMMENT |
| --- | --- | --- | --- | --- | --- |
| 4100 | 90,45,00 |  | MOV | DPTR,#4500 | Initialize memory location |
| 4103 | 74,FF |  | MOV | A,#FF | Get the data in accumulator |
| 4105 | D3 |  | SETB | C | Set CY bit |
| 4016 | 82,EF |  | ANL | C, ACC.7 | Perform AND with 7th bit of accumulator |
| 4018 | E5,D0 |  | MOV | A,DOH | Store the result |
| 410A | F0 |  | MOVX | @DPTR,A |  |
| 410B | A3 |  | INC | DPTR | Go to next location |
| 410C | 72,AA |  | ORL | C, IE.2 | OR CY bit with 2nd bit if IE reg |
| 410E | C2,D6 |  | CLR | PSW.6 | Clear 6th bit of PSW |
| 4110 | E5,D0 |  | MOV | A,DOH | Store the result |
| 4112 | F0 |  | MOVX | @DPTR,A |  |
| 4113 | A3 |  | INC | DPTR | Go to next location |
| 4114 | D2,90 |  | SETB | SCON.5 | Set 5th of SCON reg |
| 4116 | C2,99 |  | CLR | SCON.1 | Clear 1st bit of SCON reg |
| 4118 | E5,98 |  | MOV | A,98H | Store the result |
| 411A | F0 |  | MOVX | @DPTR,A |  |
| 411B | A3 |  | INC | DPTR | Go to next location |
| 411C | A2,99 |  | MOV | C,SCON.1 | Copy 1st bit of SCON reg to CY flag |
| 411E | E5,D0 |  | MOV | A,DOH | Store the result |
| 4120 | F0 |  | MOVX | @DPTR,A |  |
| 4122 | 80,FE | L2 | SJMP | L2 | Stop |

| MEMORY LOCATION | SPECIAL FUNCTION REGISTER FORMAT |  |  |  |  |  |  |  | BEFORE EXECUTION | AFTER EXECUTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4500H (PSW) | CY | AC | FO | RS1 | RS0 | OV | - | P | 00H | 88H |
| 4501H (PSW) | CY | AC | FO | RS1 | RS0 | OV | - | P | 40H | 88H |
| 4502H (SCON) | SM0 | SM1 | SM2 | REN | TB8 | RB8 | TI | RI | 0FH | 20H |
| 4503H (PSW) | CY | AC | FO | RS1 | RS0 | OV | - | P | FFH | 09H |

RESULT: Thus the bit manipulation operation is done in 8051 microcontroller.

EX.NO	PROGRAMS TO VERIFY TIMER, INTERRUPTS & UART OPERATIONS IN 8031 MICROCONTROLLER

DATE :

- Program to generate a square wave of frequency --------.

Steps to determine the count:

Let the frequency of sqaurewave to be generated be Fs KHz.

And the time period of the squarewave be Ts Sec.

Oscillator Frequency = 11.0592MHz.

One machine cycle = 12 clock periods

Time taken to complete one machine cycle=12*(1/11.0592MHz)= 1.085microsec.

Y(dec) = (Ts/2)/(1.085microsec)

Count(dec) = 65536(dec) – Y(dec)

= Count(hexa)

MOV TMOD,#10h	; To select timer1 & mode1 operation

L1:		MOV TL1,#LOWERORDER BYTE OF THE COUNT

MOV TH1,#HIGHER ORDER BYTE OF THE COUNT

SETB TR1		; to start the timer (TCON.6)

BACK:	JNB TF1,BACK	; checking the status of timerflag1(TCON.7) for

overflow

CPL Px.x		; get the square wave through any of the portpins

; eg. P1.2 (second bit of Port 1)

CLR TR1		; stop timer

CLR TF1		; clear timer flag for the next cycle

SJMP L1

- Program to transfer a data serially from one kit to another.

Transmitter:

MOV TMOD,#20H		; Mode word to select timer1 & mode 2

MOV TL1,#FDH		; Initialize timer1 with the count

MOV TH1,#FFH

MOV SCON,#50H		; Control word for serial communication to

to select serial mode1

SETB TR1			; Start timer1

MOV A,#06h

MOV SBUF,A		; Transfer the byte to be transmitted to serial

Buffer register.

LOOP:		JNB TI, LOOP		; checking the status of Transmit interrupt

flag

CLR TI

HERE:		SJMP HERE

Receiver:

MOV TMOD,#20H

MOV TL1,#FDH

MOV TH1,#FFH

MOV SCON,#50H

SETB TR1

LOOP:		JNB RI,LOOP

MOV A,SBUF

MOV DPTR,#4500H

MOVX @DPTR,A

CLR RI

HERE:		SJMP HERE

EX.NO. 	COMMUNICATION BETWEEN 8051 MICROCONTROLLER

KIT & PC

DATE :

SERIAL COMMUNICATION

8051>H

HELP MENU

D	Display data, program, internal, bit memory or registers

E 	Edit data, program, internal, bit memory or registers

S 	Single step from specified address, press SP to terminate

G 	Execute the program till user break

B 	Set address till where the program is to be executed

C	Clear break points

F10	Key followed by 4 key at the PC to upload data to a file (DOS)

T	Test the onboard peripherals

:	Download a file from PC mem to the SDA-SI-MEL kit (DOS)

A	Assembler

Z 	Disassembler

TEST FOR ONBOARD PERIPHERALS

For SDA SI-MEL kit, following menu is displayed on pressing the option "T"

8051>T

ALS-SDA SI-MEL Kit Test monitor

- Test internal Data RAM

- Test external Data Memory (U6)

- Test external Data memory (U7)

- 8255 loop test

- Test 8253

- Exit

Select (1-6):

Suppose the user presses the key '1', following message is displayed if the internal data RAM is OK.

Testing internal data RAM: Pass

After displaying the message, the menu is displayed once again waits for user to enter a key

EDITING MEMORY COMMAND:

8051>E

EDIT (R,B,M,P,D)…D - EXTERNAL DATA RAM

Enter STA address = 0400

0400 = 	7F:55	Press 'N' key to go to the next address

0401 = 	D5:66

0402 = 	D3:77

0403 =		73:88

0404 =		6F:12

0405 =		CB:01

0406 =		A7:02	Press 'P' key to go to the previous address

0407 = 	6F:03

0408 = 	7B:04

0409 = 	29:05

040A = 	6F:06

040B = 	73:07

040C =		FF:08

040D =	7D:09	Press 'CR' key to have the same address

040E = 	09:90	Press 'ESC' Key to abort the command

8051>E

EDIT (R,B,M,P,D)…B - BITS

Enter STA address = 00

00 = 0:1

01= 0:1

02 = 0:0

03 = 0:1

03 = 1:

03 = 1:

02 = 0:

8051>E

EDIT (R,B,M,P,D)…R- REGISTERS

ACC = 00:33

PSW = 00:44

DPH = 00:55

DPL = 00:00

DPL = 00:00

8051>E

EDIT (R,B,M,P,D)…-P = PROGRAM CODE

8000 = FF:78

8001 = FF:10

8002 = FF:79

8003 = FF:20

8004 = FF:7A

8005 = FF: 12

- = FF : 00

- = FF : 03

- = FF : 0F

8051>E

EDIT (R,B,M,P,D)…-M - INTERNAL RAM

0000 = 00 : 12

0001 = 00 : 34

0002 = 00 : 00

DISPLAY COMMAND

8051>D

EDIT (R,B,M,P,D)…-EXTERNAL DATA RAM

Enter STA address = 0400

Enter END address = 040F

0500 55 66 77 88 12 01 02 03 04 05 06 07 08 09 04 D7

SETTING BREAK COMMAND :

8051>B

BR _ NO: R

BR_ADD 0000

ERROR! ONLY A BREAKS ALLOWED

8051>B

BR _ NO: 0

ERROR! BREAK NUMBERS MUST BE BETWEEN 1 & 8

CLEAR BREAK COMMAND:

8051>C

BR_N0:A	Clears all the break point set by the user

8051>C

BR_N0:1	Clears the break point number 1

PROGRAMME EXECUTION COMMAND:

8051>G

PROGRAM EXECUTION

ENTER START ADDRESS = 8000

ACC PSW DPH DPL PCH PCL SP B R0 R1 R2 R3 R3 R4 R5 R6 R7

33 44 55 00 10 34 00 00 00 00 00 00

ASSEMBLE MEMORY COMMAND

8051>A

ENTER START ADDRESS = 8000

DISASSEMBLE MEMORY COMMAND

8051>Z

EX. NO. 	PROGRAMS FOR DIGITAL CLOCK AND STOP WATCH (USING 		8086)

DATE:

Program that places a message on the screen every 10 seconds, using int 1ah;

CODE SEGMENT

TIMEDELAY:

MOV SP,1000H

MOV DI,10XD

TIME OUT:

MOV AH,00H

INT 1AH

MOV BX,DX

TIMER:

MOV AH, 00H

INT 1AH

SUB DX, BX

CMP DX, 182XD

JC TIMER

MOV AH, 09H

CS 	MOV DX,MSG

INT 21H

DEC DI

JNZ TIMEOUT

MOV AX,4C00H

INT 21H

MSG:

DB 'TEN MORE SECONDS HAVE PASSED $'

CODE ENDS

EXPERIMENTS USING MASM – 8086 PROGRAMS

Simple Arithmetic Operation

16-BIT ADDITION

PROBLEM STATEMENT:

Write a program to add the given two 16-bit Nos. in 8086p.

ALGORITHM:

1 Get the addend and augend.

2. Initialize DX register for carry

3. Add addend and augend.

4. If there is carry, increment DX register and go to step6 or else directly go to step6.

5. Initialize the memory pointer to output location

6. Store the result & carry in consecutive memory locations.

7. Stop the program execution.

EXERCISE:

Write an ALP using INTEL8086 mnemonics to add any two 32-bit numbers.

PROGRAM:

data_here       segment

firstno         dw      0202h		; first no.

secondno     dw      0202h		; second no.

sum           dw      2 dup(0)		; store sum here

data_here       ends

code_here      segment

assume cs:code_here,ds:data_here

start:      mov ax,data_here			; initialize data segment

mov ds,ax

mov ax,firstno				; get first no.

mov dx,0000h				; initialize dx for carry.

add ax,secondno			; add second to it.

jnc go

inc dx

go:         mov sum,ax				; store the sum & carry.

mov sum+2,dx

int 3

code_here      ends

end start

FLOWCHART:

START

AX 
 Addend

DX
 0000H

AX
AX + Second No.

Is Carry flag set?

DX
DX+1

[Sum] 
 AX

[Sum+2] 
DX

STOP

NO

YES

#### II. 16-BIT SUBTRACTION

PROBLEM STATEMENT:

Write a program to subtract given two, 16 bit numbers.

ALGORITHM:

- Get the minuend and subtrahend.

- Compare the minuend and subtrahend. If minuend is lesser than subtrahend, interchange the numbers and increment Dx register.

- Subtract subtrahend from minuend.

- Initialize the memory pointer to output memory location.

- Store the results in two memory locations and DX register content in the next memory location.

- Stop the program execution.

CONCLUSION:

Thus, subtraction of two 16-bit numbers was performed.

EXERCISE:

- Write an ALP to subtract any two 32-bit numbers using INTEL8086 mnemonics.

- Write an ALP to subtract any two 16-bit numbers.

(HINT: If subtrahend is greater than minuend, take 2’s complement of the result and indicate it by putting 01 in DL register.)

PROGRAM:

data_here       segment

minuend         dw      2222h		; Minuend

subtrahend      dw      1111h		; Subtrahend

result          dw      2 dup(0)		; Store result here.

data_here       ends

code_here       segment

assume cs:code_here,ds:data_here

start:       mov ax,data_here			; Initialize data segment.

mov ds,ax

mov ax,minuend			; Get minuend & store in Acc.

mov dx,subtrahend			; Get subtrahend & store in dx.
                       mov cx,0000h				; Initialize cx for carry

cmp ax,dx				; compare minuend & subtrahend if

jnc ahead				  minuend smaller than subtrahend ,

mov bx,dx				  interchange minuend & subtrahend.

mov dx,ax

mov ax,bx

mov cx,0001h				; Increment carry by one.

ahead:     sub ax,dx				; subtract dx from ax

mov result,ax				; store the result & carry.

mov result+2,cx

int 3

code_here       ends

end start

FLOWCHART:

START

STOP

AX   
 Minuend

DX
 Subtrahend

CX
 0000H

Is

DX>AX ?

BX
DX

DX
AX

AX
BX

CX
01H

AX
AX - DX

[Result] 
 AX

[Result+2] 
 CX

YES

NO

#### III. 16-BIT MULTIPLICATION

PROBLEM STATEMENT:

Write a program to multiply two, 16-bit numbers using MASM software.

ALGORITHM:

- Get the multiplicand and multiplier

- Multiply the multiplicand with multiplier using repeated addition method.

- Initialize the memory pointer to output memory location.

- Store the results in memory locations.

- Stop the program execution.

EXERCISE:

Write an ALP using INTEL8086 mnemonics to multiply two signed 16-bit numbers.

FLOWCHART:

START

AX  
    Multiplicand

DX, AX
AX . Multiplier

[Product] 
 AX

[Product+2] 
DX

STOP

PROGRAM:

data_here	segment

multiplicand    dw      0202h		; Multiplicand

multiplier      dw      0202h		; Multiplier

product         dw      2 dup(0)		; store product here.

data_here       	ends

code_here       	segment

assume cs:code_here,ds:data_here

start:	mov ax,data_here			; Initialize data segment.

mov ds,ax

mov ax,multiplicand			; Get multiplicand

mul multiplier				; multiply multiplier with it.

mov product,ax			; Store the result.

mov product+2,dx

int 3

code_here       ends

end start

CONCLUSION:

Thus, multiplication of two, 16-bit numbers is performed using INTEL 8086 Mnemonics.

#### IV. 16-BIT DIVISION

PROBLEM STATEMENT:

Write a program to Divide two, 16-bit numbers using MASM software.

ALGORITHM:

- Get the dividend and divisor.

- Divide dividend by divisor.

- Initialize the memory pointer to output memory location.

- Store the results in memory locations.

- Stop the program execution.

EXERCISE:

Write an ALP using INTEL8086 mnemonics to divide two signed 16-bit numbers.

FLOWCHART:

START

AX    
Dividend

DX
0000H

AX / Divisor

AX
Quotient

DX
Remainder

[Product] 
 AX

[Product+2]
DX

STOP

PROGRAM

data_here	segment

dividend        dw      2222h		; Dividend

divisor         dw      1111h		; Divisor

result          dw      2 dup(0)		; Store result here.

data_here 	 ends

code_here	segment

assume cs:code_here,ds:data_here

start:	mov ax,data_here			; Initialize data segment.

mov ds,ax

mov ax,dividend			; Get dividend

div divisor				; Divide it by divisor.

mov result,ax				; Store result.

mov result+2,dx

int 3

code_here        ends

end start

CONCLUSION:

Thus, division of two, unsigned 16-bit numbers is performed using INTEL 8086 Mnemonics.

12 . BIOS/DOS CALLS – DISPLAY

AIM:

To display a message on the CRT screen of a microcomputer using DOS calls.

ALGORITHM:

- Initialize the data segment and the message to be displayed.

- Set function value for display.

- Point to the message and run the interrupt to display the message in the CRT.

PROGRAM:

ASSUME CS: CODE, DS: DATA

DATA SEGMENT

MSG DB 0DH, 0AH, “GOOD MORNING” , ODH, OAH, “$”

DATA ENDS

CODE SEGMENT

START:	MOV AX, DATA

MOV DS, AX

MOV AH, 09H

MOV DX, OFFSET MSG

INT 21H

MOV AH, 4CH

INT 21H

CODE ENDS

END START

RESULT:

A message is displayed on the CRT screen of a microcomputer using DOS calls

13. BIOS/DOS CALLS – FILE MANIPULATION

AIM:

To open a file using DOS calls.

ALGORITHM:

- Initialize the data segment, file name and the message to be displayed.

- Set the file attribute to create a file using a DOS call.

- If the file is unable t o create a file display the message

PROGRAM:

ASSUME CS: CODE, DS: DATA

DATA SEGMENT

FILENAME DB “SAMPLE.DAT”, “$”

MSG DB 0DH, 0AH, “FILE NOT CREATED”, ODH, OAH, “$”

DATA ENDS

CODE SEGMENT

START:	MOV AX, DATA

MOV DS, AX

MOV DX, OFFSET FILENAME

MOV CX, 00H

MOV AH, 3CH

INT 21H

JNC LOOP1

MOV AX, DATA

MOV DS, AX

MOV DX, OFFSET MSG

MOV AH, 09H

INT 21H

LOOP1	MOV AH, 4CH

INT 21H

CODE ENDS

END START

RESULT  :         A file is opened using DOS calls.

14. BIOS/DOS CALLS – DISK INFORMATION

AIM:

To display the disk information.

ALGORITHM:

- Initialize the data segment and the message to be displayed.

- Set function value for disk information.

- Point to the message and run the interrupt to display the message in the CRT.

PROGRAM:

ASSUME CS: CODE, DS: DATA

DATA SEGMENT

MSG DB 0DH, 0AH, “GOOD MORNING” , ODH, OAH, “$”

DATA ENDS

CODE SEGMENT

START:	MOV AX, DATA

MOV DS, AX

MOV AH, 36H

MOV DX, OFFSET MSG

INT 21H

MOV AH, 4CH

INT 21H

CODE ENDS

END START

RESULT:

The disk information is displayed.

15 STRING MANIPULATION

I. 8086 STRING MANIPULATION – SEARCH A WORD

AIM:

To search a word from a string.

ALGORITHM:

  - Load the source and destination index register with starting and the ending address respectively.

  - Initialize the counter with the total number of words to be copied.

  - Clear the direction flag for auto incrementing mode of transfer.

  - Use the string manipulation instruction SCASW with the prefix REP to search a word from string.

  - If a match is found (z=1), display 01 in destination address. Otherwise, display 00 in destination address.

RESULT:

A word is searched and the count of number of appearances is displayed.

PROGRAM:

ASSUME CS: CODE, DS: DATA

DATA SEGMENT

LIST DW 53H, 15H, 19H, 02H

DEST EQU 3000H

COUNT EQU 05H

DATA ENDS

CODE SEGMENT

START:	MOV AX, DATA

MOV DS, AX

MOV AX, 15H

MOV SI, OFFSET LIST

MOV DI, DEST

MOV CX, COUNT

MOV AX, 00

CLD

REP		SCASW

JZ LOOP

MOV AX, 01

LOOP		MOV [DI], AX

MOV AH, 4CH

INT 21H

CODE ENDS

END START

INPUT:

LIST: 53H, 15H, 19H, 02H

OUTPUT:	3000	01

II.8086 STRING MANIPULATION –FIND AND REPLACE A WORD

AIM:

To find and replace a word from a string.

ALGORITHM:

- Load the source and destination index register with starting and the ending address respectively.

- Initialize the counter with the total number of words to be copied.

- Clear the direction flag for auto incrementing mode of transfer.

- Use the string manipulation instruction SCASW with the prefix REP to search a word from string.

- If a match is found (z=1), replace the old word with the current word in destination address. Otherwise, stop.

RESULT:

A word is found and replaced from a string.

PROGRAM:

ASSUME CS: CODE, DS: DATA

DATA SEGMENT

LIST DW 53H, 15H, 19H, 02H

REPLACE EQU 30H

COUNT EQU 05H

DATA ENDS

CODE SEGMENT

START:	MOV AX, DATA

MOV DS, AX

MOV AX, 15H

MOV SI, OFFSET LIST

MOV CX, COUNT

MOV AX, 00

CLD

REP		SCASW

JNZ LOOP

MOV DI, LABEL LIST

MOV [DI], REPLACE

LOOP		MOV AH, 4CH

INT 21H

CODE ENDS

END START

INPUT:

LIST: 53H, 15H, 19H, 02H

OUTPUT:

LIST: 53H, 30H, 19H, 02H

III. 8086 STRING MANIPULATION – COPY A STRING

AIM:

To copy a string of data words from one location to the other.

ALGORITHM:

  - Load the source and destination index register with starting and the ending address respectively.

  - Initialize the counter with the total number of words to be copied.

  - Clear the direction flag for auto incrementing mode of transfer.

  - Use the string manipulation instruction MOVSW with the prefix REP to copy a string from source to destination.

RESULT:

A string of data words is copied from one location to other.

PROGRAM:

ASSUME CS: CODE, DS: DATA

DATA SEGMENT

SOURCE EQU 2000H

DEST EQU 3000H

COUNT EQU 05H

DATA ENDS

CODE SEGMENT

START:	MOV AX, DATA

MOV DS, AX

MOV ES, AX

MOV SI, SOURCE

MOV DI, DEST

MOV CX, COUNT

CLD

REP	MOVSW

MOV AH, 4CH

INT 21H

CODE ENDS

END START

INPUT:			OUTPUT:

2000 	48			3000	48

2001	84			3001	84

2002 	67			3002	67

2003	90			3003	90

2004	21			3004	21

IV.8086 STRING MANIPULATION – SORTING

AIM:

To sort a group of data bytes.

ALGORITHM:

- Place all the elements of an array named list (in the consecutive memory locations).

- Initialize two counters DX & CX with the total number of elements in the array.

- Do the following steps until the counter B reaches 0.

  - Load the first element in the accumulator

  - Do the following steps until the counter C reaches 0.

- Compare the accumulator content with the next element present in the next memory location. If the accumulator content is smaller go to next step; otherwise, swap the content of accumulator with the content of memory location.

- Increment the memory pointer to point to the next element.

- Decrement the counter C by 1.

  - Stop the execution.

RESULT:

A group of data bytes are arranged in ascending order.

PROGRAM:

ASSUME CS: CODE, DS: DATA

DATA SEGMENT

LIST DW 53H, 25H, 19H, 02H

COUNT EQU 04H

DATA ENDS

CODE SEGMENT

START:	MOV AX, DATA

MOV DS, AX

MOV DX, COUNT-1

LOOP2:	MOV CX, DX

MOV SI, OFFSET LIST

AGAIN:	MOV AX, [SI]

CMP AX, [SI+2]

JC LOOP1

XCHG [SI +2], AX

XCHG [SI], AX

LOOP1:	ADD SI, 02

LOOP AGAIN

DEC DX

JNZ LOOP2

MOV AH, 4CH

INT 21H

CODE ENDS

END START

INPUT:

LIST: 53H, 25H, 19H, 02H

OUTPUT:

LIST: 02H, 19H, 25H, 53H